Q1. Debugging Overflow in 8-bit 2’s Complement Addition:

Given: +120 + +50

# Step 1: Convert to 8-bit binary

+120 = 01111000

+50  = 00110010

# Step 2: Perform addition

01111000

+ 00110010

-----------   10101010

The result 10101010 has MSB = 1, so it represents a negative number in 2’s complement.

Its value is:

10101010 → 01010101 → 10101010

Thus, the binary result does not represent the expected +170.

# Step 3: Identify overflow

The range of signed 8-bit numbers is:

−128 to +127 But:

120 + 50 = 170

Since +170 is outside this range, overflow has occurred.

Overflow occurs when:

 Two positive numbers produce a negative result, or   Two negative numbers produce a positive result.

# Solution

Use an overflow detection condition:

Overflow = Carry into MSB XOR Carry out of MSB

The processor should set an overflow flag and handle the result appropriately instead of treating 10101010 as a valid positive result.

Conclusion:

The error is caused by exceeding the 8-bit signed range. Increasing the bit width or detecting the overflow flag solves the problem.

Q2. Ripple Carry Adder Delay and Carry Look-Ahead Solution:

# Problem

In a Ripple Carry Adder (RCA), each carry must wait for the previous carry.

For example:

C0 → C1 → C2 → C3 → C4

Therefore, the carry generated at the first bit may need to travel through all bits before the final sum is obtained.

# Problem Analysis

For a 4-bit RCA:

Bit 0 → Bit 1 → Bit 2 → Bit 3

Each full adder waits for the previous carry. Hence, the propagation delay increases with the number of bits.

# Optimized Solution: CLA

A Carry Look-Ahead Adder calculates carries in advance.

Generate: Gi = Ai · Bi Propagate: Pi = Ai  Bi

Carry:

Ci+1 = Gi + PiCi For example:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

Thus, carries are calculated in parallel instead of waiting for one another.

# Advantages

- Reduces carry propagation delay.

- Faster arithmetic operations.

- Suitable for real-time and high-performance processors.

- More efficient for larger bit-width operations.

Conclusion:

Replacing RCA with CLA significantly reduces the delay because carry signals are calculated in advance.

Q3. Debugging Booth’s Algorithm for Negative Operands:

Booth’s algorithm is used for signed binary multiplication using 2’s complement.

# Common Errors

Incorrect results may occur because of:

- Wrong checking of the pair (Q0, Q−1).

- Performing a logical shift instead of an arithmetic right shift.

- Not maintaining the sign bit during shifting.

- Incorrect 2’s complement representation of negative numbers.

- Incorrect addition/subtraction of the multiplicand.

# Booth Rules

Q0 Q−1 Operation

00 	No operation

11 	No operation

01 	A = A + M

10 	A = A − M

After every operation, perform an arithmetic right shift on:

A | Q | Q−1 Debugging

- Check that negative operands are represented using 2’s complement.

- Check both Q0 and Q−1 before every operation.

- Preserve the sign bit during arithmetic shifting.

- Use the same bit width for the accumulator and multiplicand.

# Correction

Use sign extension when storing negative values and always perform an arithmetic right shift, where the MSB is copied into the new MSB position.

Conclusion: Most Booth multiplication errors are caused by incorrect bit-pair checking, sign extension, or shifting. Correcting these steps gives accurate signed multiplication.

Q4. Restoring Division and Non-Restoring Optimization:

# Problem

In the restoring division algorithm, after subtracting the divisor, if the partial remainder becomes negative, the divisor is added back.

This is called restoration.

R = R − M If R < 0:

R = R + M

Repeated restoration causes:

- Extra addition operations.

- Increased execution time.

- More hardware activity.

# Optimized Solution: Non-Restoring Division

Non-restoring division avoids immediate restoration.

- If the remainder is positive, subtract the divisor.

- If the remainder is negative, add the divisor in the next iteration.

- At the end, correct the remainder if necessary.

# Comparison

| Restoring | Non-Restoring |
| --- | --- |
| Restores negative remainder | Does not immediately restore |
| More operations | Fewer operations |
| Higher execution time | Lower execution time |
| Simpler | More efficient |

# Conclusion

Non-restoring division improves performance because it eliminates repeated restoration operations. It is therefore useful in faster processor and embedded-system arithmetic units.

Q5. IEEE 754 Floating-Point Accuracy Problem:

# Problem

When adding a very large number and a very small number using single precision, the smaller number may lose precision.

# Step 1: Exponent Alignment

Before addition, exponents must be made equal.

For example:

Large number: 1.234 × 2¹ Small number: 1.100 × 2²

The smaller number is shifted to match the exponent of the larger number.

After alignment, the small number may become too small to affect the significant bits.

# Step 2: Normalization

After addition, the result is normalized into the form:

1.xxxxx × 2ⁿ

# Step 3: Rounding

If the result contains more bits than the available 23-bit fraction field, extra bits are rounded.

This can cause rounding error and precision loss.

# Solutions

- Use double precision (64-bit) instead of single precision.

- Use numerically stable algorithms.

- Avoid repeatedly adding numbers with very different magnitudes.

- Use techniques such as compensated summation when many values are added.

- Normalize and round carefully according to IEEE 754 rules.

Conclusion:

The inaccuracy occurs mainly because single precision has limited precision. Using double precision and suitable numerical algorithms can significantly reduce the error.

Code of Conduct:

I ________________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctnes s of Debugged | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Solution (25%) |  |  |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |