SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

Cache memory is a small, high-speed memory located close to the CPU. Modern processors generally use multiple cache levels so that frequently accessed instructions and data can be supplied to the processor faster than accessing main memory. L1 is the closest and lowest-latency level, while L3 is larger but slower. Intel describes L1 as the shortest-latency cache and L3/LLC as the last cache level before DRAM.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU core | Near/within core | Usually shared among cores |
| Typical size | Smallest | Medium | Largest |
| Relative latency | Lowest | Higher than L1 | Higher than L2 |
| Relative bandwidth | Highest | Very high | Lower than L1/L2 |
| Access purpose | Immediate data/instructions | Backup for L1 | Last cache before DRAM |
| Typical role | Very frequently used data | Larger working set | Shared/common working data |
| Performance effect | Very high | High | Important for reducing DRAM accesses |

An Intel reference gives an illustrative configuration of approximately 1 ns and 1 TB/s for L1, 4 ns and 1 TB/s for L2, and more than 400 GB/s for L3, while emphasizing that actual values vary by processor and workload.

### Impact on processor performance

A processor first checks L1. If the requested data is not found, it checks L2, followed by L3, and finally main memory. Therefore, a high L1 hit rate provides the greatest performance benefit because the processor can continue execution with minimal waiting. Intel notes that avoiding L1, L2, and LLC misses improves latency and processor performance.

L2 provides a larger backup area, reducing the number of accesses that must reach L3 or DRAM. L3 is useful for larger working sets and, on many processors, allows multiple cores to share cached data. Intel documentation gives examples where L1 and L2 are private to each core while the LLC is shared.

### Critical analysis

The hierarchy represents a trade-off:

L1 → smallest + fastest + highest bandwidth
L2 → larger + moderately slower
L3 → largest + slower but useful for reducing DRAM accesses

Increasing cache capacity usually requires more area and power, so making every cache extremely large is not practical. The multi-level approach therefore provides a good balance between latency, capacity, bandwidth, cost, and power.

### Conclusion

L1 has the greatest effect on immediate CPU performance because of its very low latency. L2 improves the hit rate beyond L1, while L3 reduces expensive main-memory accesses. Efficient cache utilization and high cache-hit rates are therefore essential for achieving high processor throughput.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

SRAM and DRAM are both volatile semiconductor memories, but their circuit structures and performance characteristics are different. SRAM is generally faster and is therefore suitable for cache memory, while DRAM provides much higher capacity at lower cost and is therefore normally used as main memory.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static Random Access Memory | Dynamic Random Access Memory |
| Storage method | Flip-flop based cell | Capacitor-based cell |
| Refresh | Not required | Required periodically |
| Latency | Very low | Higher |
| Bandwidth | Very high for cache access | High, but generally slower in access latency |
| Throughput | Excellent for CPU-cache workloads | Excellent for large-capacity memory workloads |
| Density | Lower | Higher |
| Cost per bit | Higher | Lower |
| Typical use | L1/L2 cache | Main memory |
| Power characteristics | No refresh power, but larger cell area | Requires refresh operations |

SRAM can respond rapidly because it does not require the periodic refresh associated with DRAM. Its larger cell size makes it more expensive per bit, however. DRAM uses compact cells, giving it much greater density and making it appropriate for large main-memory systems.

### SRAM in cache design

Cache memory must respond rapidly because the CPU may request data every clock cycle. Therefore, SRAM is preferred for processor caches. Intel identifies L1 as the shortest-latency level and shows that low-level caches are central to reducing CPU waiting time.

The small capacity of SRAM is acceptable because caches store only frequently or recently used data.

### DRAM in main memory

DRAM is economical and dense, allowing computers to have gigabytes of main memory. Its access latency is greater than SRAM, but its large capacity makes it suitable for storing programs and data that cannot fit in cache.

Intel's illustrative memory hierarchy reports approximately 100 GB/s for main memory, compared with much greater bandwidth for the processor cache levels in that example.

### Critical analysis

SRAM and DRAM should not be viewed as competing technologies for exactly the same purpose. They are complementary:

SRAM → speed and low latency
DRAM → capacity and cost efficiency

Using SRAM for all system memory would make a computer extremely expensive and area-intensive. Using DRAM for CPU cache would increase processor waiting time.

### Conclusion

SRAM is better suited to cache because of its low latency and high-speed operation. DRAM is better suited to main memory because of its high density, lower cost per bit, and sufficient bandwidth for large-scale data storage. The combination of both creates an effective memory hierarchy.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

Virtual memory and cache memory both improve computer-system memory management, but they solve different problems. Cache memory reduces the average time required to access frequently used information, while virtual memory allows programs to use an address space larger than the available physical RAM.

Paging is the mechanism commonly used to implement virtual memory. A virtual address is divided into components, and page tables map virtual pages to physical page frames. IBM explains that virtual-memory pages may reside in RAM or in paging space on disk, and that page tables map virtual addresses to physical locations.

| Feature | Cache Memory | Virtual Memory with Paging |
| --- | --- | --- |
| Main purpose | Reduce memory-access latency | Extend available address space |
| Basic unit | Cache line | Page |
| Managed mainly by | Hardware/cache controller | OS + MMU |
| Fast path | Cache hit | Page already in RAM |
| Slow path | Cache miss → lower cache/memory | Page fault → storage access |
| Typical latency | Very low | Much higher during page fault |
| Effect on throughput | Usually improves throughput | Can reduce throughput if excessive paging occurs |
| Capacity | Small | Very large logical address space |
| Typical technology | SRAM | DRAM + SSD/HDD paging storage |

### Cache mechanism

When the processor requests data, the cache is searched first. If the data is present, a cache hit occurs and the CPU receives it quickly. If it is absent, a cache miss causes the processor to obtain the data from a lower level.

High cache-hit rates improve system throughput because fewer processor cycles are lost waiting for DRAM.

### Paging mechanism

Paging divides virtual memory into fixed-size pages and physical memory into page frames. When a required page is not present in RAM, a page fault occurs and the operating system must retrieve the page from secondary storage. IBM states that paging becomes substantially more expensive than using RAM and that excessive paging can cause thrashing.

IBM documentation gives an example in which a page fault can take around 10 ms or more, compared with a cache miss taking only a few dozen CPU cycles. The exact values depend on the system, but the comparison shows the enormous performance difference between cache misses and storage-backed page faults.

### Throughput analysis

Cache memory generally increases throughput by supplying frequently used data quickly. Virtual memory increases the number and size of programs that can run, but excessive paging can reduce throughput considerably.

Therefore:

Cache → performance optimization

Virtual memory → capacity and memory-isolation mechanism

### Critical analysis

A system can benefit from both mechanisms simultaneously. For example, a processor may access a virtual address, translate it through the memory-management system, find the required page in DRAM, and then use processor caches to reduce subsequent access latency.

However, a high page-fault rate can dominate performance even when the cache is well designed. IBM describes continuous page-in/page-out behavior as thrashing, where most system time is spent moving pages instead of performing useful computation.

### Conclusion

Cache memory primarily improves access latency and throughput, whereas virtual memory with paging primarily improves address-space capacity and process management. Cache misses are relatively inexpensive compared with page faults, so efficient paging and sufficient RAM are essential for maintaining high system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

NAND Flash and DRAM are fundamentally different memory technologies. DRAM is volatile and designed for fast random-access working memory, whereas NAND Flash is non-volatile and is widely used for persistent storage such as SSDs.

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |
| Access behavior | Fast random access | Page-oriented read/write with erase constraints |
| Latency | Low | Much higher than DRAM for memory access |
| Bandwidth | High | High sequential bandwidth possible, but access behavior differs |
| Write behavior | Fast memory writes | Program/erase operations are slower |
| Refresh | Required | Not required |
| Density | High | Very high |
| Cost per bit | Higher than NAND | Lower |
| Typical location | Main memory | Secondary storage |
| Role in hierarchy | Working memory | Persistent storage |

DRAM sits relatively close to the CPU in the memory hierarchy. NAND Flash is normally farther away because its access latency is much higher, even though modern SSDs can provide very high sequential bandwidth.

### Latency comparison

DRAM is designed for relatively fast access to actively used program data. NAND Flash is optimized more for persistent storage capacity and cost efficiency than for CPU-style random memory access.

When a program experiences a page fault and the required data must be read from storage, the CPU may wait much longer than it would for an ordinary DRAM access. IBM notes that storage-backed virtual-memory accesses can be orders of magnitude slower than ordinary RAM accesses.

### Bandwidth comparison

Modern NAND-based SSDs can provide substantial sequential read/write bandwidth. However, bandwidth alone does not make NAND Flash suitable as a direct replacement for DRAM. Random-access latency, write/erase behavior, and persistence characteristics must also be considered.

DRAM is designed to serve as a continuously accessible working memory, whereas NAND Flash is better suited for storing large amounts of persistent information.

### Suitability in the memory hierarchy

A simplified hierarchy is:

CPU Registers → L1/L2/L3 Cache → DRAM → NAND Flash Storage

The farther a technology is from the CPU, the greater its capacity and generally its access latency. This arrangement balances speed and cost.

### Critical analysis

Replacing all DRAM with NAND Flash would provide greater persistence and density but would severely affect the performance of normal CPU memory operations. Conversely, using DRAM for persistent storage would increase cost and lose data when power is removed.

Thus, NAND Flash and DRAM are complementary technologies rather than direct substitutes.

### Conclusion

DRAM is appropriate for main memory because it provides fast random access and high memory bandwidth. NAND Flash is appropriate for persistent storage because it offers high density, non-volatility, and good cost efficiency. Both are important levels of a complete hierarchical memory system.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

MRAM and RRAM are emerging non-volatile memory technologies intended to combine some of the benefits of conventional memory with data retention after power is removed.

MRAM (Magnetoresistive Random Access Memory) stores information using magnetic states, while RRAM (Resistive Random Access Memory) stores information by changing the resistance of a memory cell.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Data storage | Magnetic state | Resistance state |
| Volatility | Non-volatile | Non-volatile |
| Access time | Very fast; can approach SRAM-like operation | Potentially very fast |
| Read speed | High | High potential |
| Write performance | Good, but write energy can be significant | Potentially low-energy operation |
| Throughput | High | High potential |
| Energy efficiency | Attractive because it is non-volatile | Attractive due to low-power potential |
| Endurance | Generally good | Technology-dependent |
| Main challenge | Write current/energy and scaling | Variability, endurance, reliability |
| Potential applications | Embedded memory, caches | Embedded memory, storage, neuromorphic/compute applications |

### MRAM performance

MRAM does not require refresh because data is stored magnetically. This can reduce standby-related energy compared with volatile memories.

The 2024 International Roadmap for Devices and Systems discusses STT-MRAM as a candidate for replacing SRAM in last-level-cache applications. It reports a demonstrated system-level example with sub-2.5 ns read access and sub-7.1 ns write access, while also noting challenges associated with write power at very high speeds.

This demonstrates that MRAM has potential for high-performance cache-like applications, although write energy remains an important design consideration.

### RRAM performance

RRAM uses changes in electrical resistance to represent data. Its non-volatile operation can support low standby power, and its simple cell structures offer potential advantages in density and scalability.

A 2024 review reports continued interest in RRAM because of its potential for speed, energy efficiency, non-volatility, scalability, and applications in emerging computing systems.

However, practical RRAM systems still face challenges such as device variability, endurance, switching consistency, and manufacturing integration.

### Throughput and energy comparison

Both technologies can provide high throughput because they are solid-state memories with no mechanical movement. Their non-volatility can also reduce the need for refresh operations or repeated reloading after power interruptions.

A simplified comparison is:

MRAM: very fast + non-volatile + good endurance + write-energy challenge

RRAM: high density potential + non-volatile + low-energy potential + variability/endurance challenges

### Critical analysis

Neither MRAM nor RRAM has completely replaced conventional SRAM and DRAM in all applications. The selection depends on the target system.

MRAM is especially attractive where fast non-volatile memory is required. RRAM is especially promising for high-density, low-power, and emerging computing architectures.

The technologies may eventually occupy positions between conventional cache, main memory, and persistent storage, depending on manufacturing cost and reliability.

### Conclusion

MRAM and RRAM both provide promising alternatives to traditional volatile memory. MRAM currently demonstrates strong potential for fast non-volatile memory applications, while RRAM offers attractive density and energy-efficiency possibilities. Their future use will depend on improving write energy, endurance, variability, manufacturing yield, and cost.

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |