SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

## Problem Understanding

A real-time embedded system receives data from multiple sensors. The sensors generate interrupts every 1 ms. The system must respond quickly while keeping CPU utilization below 15%. Vectored interrupt handling must be used.

### Given Constraints

- CPU utilization < 15%.

- Multiple sensors generate interrupts every 1 ms.

- Vectored interrupts are mandatory.

- I/O technique must provide low latency.

- The system must handle multiple sensors efficiently.

## Constraint Analysis

If the CPU directly polls every sensor, CPU utilization will increase. Interrupt-driven I/O is therefore preferred.

With vectored interrupts, each device or interrupt source has a predefined interrupt service routine (ISR) address. The CPU can directly execute the required ISR without searching for the source.

### Memory-Mapped vs I/O-Mapped I/O

| Feature | Memory-Mapped I/O | I/O-Mapped I/O |
| --- | --- | --- |
| Address space | Shares memory address space | Separate I/O address space |
| Instructions | Normal load/store | Special I/O instructions |
| Access flexibility | High | Lower |
| Programming | Simple | More specialized |
| Latency | Very low | Low |
| Recommendation | Preferred | Alternative |

## Proposed Architecture

Sensors → Interrupt Controller → CPU

CPU ↔ Memory Bus ↔ Sensor Registers

The sensor control/status registers are mapped into the CPU's memory address space.

### Operation

- Sensor generates an interrupt every 1 ms.

- Interrupt controller receives the request.

- Controller identifies the sensor.

- A vector corresponding to the sensor is generated.

- CPU jumps directly to the appropriate ISR.

- ISR reads the sensor register.

- Data is stored in memory.

- ISR clears the interrupt.

- CPU returns to the main program.

### Pseudocode

Initialize sensors

Initialize interrupt controller

Assign vector to each sensor

While system is running:

Execute main task

On sensor interrupt:

Identify vector

Execute corresponding ISR

Read sensor data

Store data in memory

Clear interrupt

Return

## Solution

Use memory-mapped I/O with vectored interrupts.

To maintain CPU utilization below 15%:

- Keep ISRs very short.

- Move processing to background tasks.

- Use interrupt batching where real-time requirements permit.

- Use DMA for large sensor data transfers.

- Give high-priority sensors higher interrupt priority.

## Justification

Memory-mapped I/O provides simple and fast register access. Vectored interrupts minimize interrupt-identification overhead. Short ISRs prevent the CPU from spending excessive time handling interrupts.

### Result

The proposed architecture provides low-latency sensor communication while minimizing CPU utilization and satisfying the given constraints.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

## Problem Understanding

The system requires storage throughput greater than 3 GB/s. NVMe over PCIe must be used, while the CPU should not handle bulk data movement.

### Given Constraints

- Throughput > 3 GB/s.

- NVMe over PCIe is mandatory.

- CPU should not perform bulk data transfer.

- DMA controller must be integrated.

- Interrupt notification must be provided.

## Constraint Analysis

A conventional programmed I/O system would require the CPU to repeatedly read and write storage data. This creates significant CPU overhead.

DMA solves this problem by allowing the storage device to transfer data directly between the device and main memory.

## Proposed Architecture

CPU → PCIe Root Complex → NVMe Controller

NVMe Controller ↔ DMA ↔ Main Memory

NVMe Controller → Interrupt Controller → CPU

### Data Transfer Process

- CPU creates an NVMe command.

- Command is placed in an NVMe submission queue.

- NVMe controller receives the command through PCIe.

- DMA transfers data directly between NVMe and DRAM.

- CPU does not copy bulk data.

- NVMe controller places completion information in the completion queue.

- Interrupt notification informs the CPU.

- CPU processes the completion event.

### Pseudocode

Create NVMe command

Place command in submission queue

Start DMA transfer

While transfer is active:

CPU performs other tasks

On completion interrupt:

Read completion queue

Process result

## Solution

Use:

NVMe SSD + PCIe + DMA + Interrupt Controller + DRAM

PCIe provides high-bandwidth communication, while NVMe provides an efficient storage protocol. DMA removes bulk-copy operations from the CPU.

### Throughput Consideration

For a required throughput:

Required throughput > 3 GB/s

A suitable PCIe configuration should provide sufficient effective bandwidth after protocol overhead. For example, PCIe Gen4 x4 provides substantially more than 3 GB/s of practical bidirectional bandwidth and is suitable for this requirement.

## Justification

DMA allows storage data to move directly between NVMe and DRAM. Therefore, CPU utilization is reduced while high throughput is maintained.

### Result

The proposed NVMe-over-PCIe architecture with DMA and interrupt notification satisfies the >3 GB/s throughput requirement and minimizes CPU involvement.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

## Problem Understanding

The system contains six USB peripherals and two high-speed Thunderbolt devices. It must prevent interrupt starvation, support hardware and software interrupts, and provide fair bus arbitration.

### Given Constraints

- Six USB peripherals.

- Two Thunderbolt devices.

- Avoid interrupt starvation.

- Use hardware and software interrupts.

- Ensure fair bus arbitration.

## Proposed Architecture

CPU

|

Interrupt Controller

/           \

Hardware       Software

Interrupts     Interrupts

|               |

USB Controller   OS/Driver Tasks

/ | | | | \

6 USB Devices

|

PCIe / I/O Fabric

|

Thunderbolt Controller

/       \

TB1       TB2

## Constraint Analysis

USB devices may generate frequent interrupts. If one device continuously receives service, other devices may experience starvation.

Thunderbolt devices require high-speed communication and should receive sufficient bus bandwidth without completely blocking USB devices.

## Bus Arbitration

A round-robin arbitration mechanism is recommended.

Example:

USB1 → USB2 → USB3 → USB4 → USB5 → USB6

↓

Thunderbolt

Each requesting device gets an opportunity to use the bus.

### Interrupt Handling

Hardware interrupts:

- USB controller events

- Thunderbolt device events

- Data-transfer completion

Software interrupts:

- Driver requests

- Operating-system services

- Deferred processing

- Background communication tasks

## Solution

Use:

- USB host controller for six USB devices.

- Thunderbolt controller for two high-speed devices.

- Interrupt controller with priority levels.

- Round-robin bus arbitration.

- Interrupt moderation for high-frequency devices.

- Software interrupts for deferred processing.

### Operation

- Device generates an interrupt.

- Interrupt controller receives it.

- High-priority hardware events are handled first.

- Normal USB events are serviced fairly.

- Driver processing is deferred using software interrupts where appropriate.

- Round-robin arbitration prevents one device from continuously occupying the bus.

## Justification

Round-robin arbitration provides fairness and prevents starvation. Hardware interrupts provide fast response for real-time device events, while software interrupts reduce the time spent inside hardware ISRs.

### Result

The architecture supports all eight devices, provides fair bus access, and prevents interrupt starvation.

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

## Problem Understanding

The data acquisition system continuously receives data at 500 MB/s. The CPU should perform minimal intervention. Device registers must use memory-mapped I/O.

### Given Constraints

- Continuous data stream = 500 MB/s.

- Minimal CPU intervention.

- Compare DMA and interrupt-driven I/O.

- Device registers must use memory-mapped I/O.

## DMA vs Interrupt-Driven I/O

| Feature | Interrupt-Driven I/O | DMA |
| --- | --- | --- |
| CPU involvement | High | Very low |
| Large data transfer | Inefficient | Efficient |
| Interrupt frequency | High | Low |
| CPU overhead | High | Low |
| Suitable for 500 MB/s | No | Yes |
| Recommended | No | Yes |

### Constraint Analysis

At 500 MB/s, transferring data through the CPU using an interrupt for every small block would generate a large number of interrupts.

DMA can transfer large blocks directly from the acquisition device to memory.

## Proposed Architecture

Data Acquisition Device → DMA Controller → DRAM

CPU ↔ Memory-Mapped Device Registers

DMA Controller → Interrupt Controller → CPU

### Operation

- CPU configures DMA registers using memory-mapped I/O.

- CPU specifies source, destination, and transfer size.

- Acquisition device begins data generation.

- DMA transfers data directly into a memory buffer.

- CPU continues other operations.

- DMA generates an interrupt after a block is completed.

- CPU processes the completed buffer.

- DMA starts the next buffer.

### Double Buffering

Use two buffers:

Device → DMA → Buffer A

↓

CPU processes

Device → DMA → Buffer B

↓

CPU processes

While the CPU processes one buffer, DMA fills the other.

## Solution

Use memory-mapped I/O + DMA + double buffering.

For a continuous rate of 500 MB/s, DMA should transfer sufficiently large blocks to reduce interrupt frequency.

For example, with a 1 MB DMA block:

Interrupt rate = 500 MB/s ÷ 1 MB ≈ 500 interrupts/s

This is much more manageable than generating an interrupt for every small data unit.

## Justification

DMA is clearly superior because the data rate is high and continuous. Memory-mapped I/O provides efficient access to the acquisition device's control and status registers.

### Result

The proposed DMA-based system supports the 500 MB/s continuous stream while keeping CPU intervention low.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

## Problem Understanding

The system must support nested interrupts and guarantee a maximum interrupt latency below 10 µs. High-priority devices must use vectored interrupts, while background tasks should use software interrupts.

### Given Constraints

- Nested interrupts must be supported.

- Maximum interrupt latency < 10 µs.

- High-priority devices use vectored interrupts.

- Background tasks use software interrupts.

## Proposed Architecture

High-Priority Devices

|

↓

Vectored Interrupt Controller

|

↓

CPU

↑

|

Software Interrupts

|

Background Tasks

## Interrupt Priority

| Priority | Source | Handling |
| --- | --- | --- |
| Highest | Emergency/real-time device | Vectored hardware interrupt |
| High | Sensor/device | Vectored interrupt |
| Medium | Normal I/O | Hardware interrupt |
| Low | Background processing | Software interrupt |

## Nested Interrupt Operation

- CPU executes the current task.

- High-priority interrupt arrives.

- CPU saves the current context.

- CPU obtains the interrupt vector.

- High-priority ISR executes.

- A higher-priority interrupt can preempt the current ISR.

- The higher-priority ISR completes.

- Previous ISR resumes.

- Original task resumes.

### Pseudocode

Run main task

On interrupt:

Save CPU context

Identify priority

If higher priority:

Execute ISR

Allow higher-priority interrupt

Restore context

Return

## Meeting the <10 µs Requirement

To achieve low interrupt latency:

- Use vectored interrupts.

- Use a fast interrupt controller.

- Keep ISR code short.

- Avoid lengthy computations inside ISRs.

- Use priority-based interrupt nesting.

- Defer background processing to software interrupts.

- Use DMA for large data transfers.

- Keep critical data structures in fast memory/cache.

### Interrupt Flow

Interrupt Request

↓

Priority Check

↓

Vector Lookup

↓

Context Save

↓

High-Priority ISR

↓

Context Restore

↓

Resume Previous Task

## Solution

A priority-based vectored interrupt controller with nested interrupt support is recommended. High-priority real-time devices receive dedicated vectors and immediate service. Background tasks use software interrupts so that they do not interfere with critical real-time processing.

## Justification

Vectored interrupts reduce interrupt identification time, while priority nesting ensures that urgent events can preempt lower-priority operations. Short ISRs and deferred processing help maintain the required latency below 10 µs.

### Result

The proposed mechanism provides nested interrupt support, fast response for high-priority devices, and efficient background processing while targeting the <10 µs interrupt-latency constraint.

# Overall Comparison of Solutions

| Question | Main Constraint | Recommended Technique | Main Benefit |
| --- | --- | --- | --- |
| Q1 | CPU < 15%, 1 ms interrupts | Memory-mapped I/O + vectored interrupts | Low latency |
| Q2 | >3 GB/s storage | NVMe + PCIe + DMA | High throughput, low CPU load |
| Q3 | 6 USB + 2 Thunderbolt | Priority interrupts + round-robin arbitration | Fairness, no starvation |
| Q4 | 500 MB/s continuous stream | Memory-mapped I/O + DMA + double buffering | Minimal CPU intervention |
| Q5 | <10 µs latency | Nested vectored interrupts + software interrupts | Fast priority response |

# Conclusion

The five designs demonstrate that I/O architecture must be selected according to system constraints.

Vectored interrupts are suitable for low-latency real-time events because the CPU can directly locate the required ISR. Memory-mapped I/O provides convenient and efficient access to device registers. DMA is essential for high-speed data transfers because it allows devices to transfer data directly to memory without continuous CPU intervention. PCIe and NVMe provide high-speed storage communication, while USB and Thunderbolt support multiple peripheral devices. Finally, nested priority-based interrupts ensure that critical events receive immediate attention without being blocked by lower-priority tasks.

Therefore, the proposed architectures satisfy the given constraints while improving CPU utilization, throughput, latency, fairness, and overall system performance.

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (25%) | Constraint Analysis (25%) | Solution Development (25%) | Application of Concepts (15%) | Justification (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |