SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

Aim: To transfer data from an I/O device to memory using DMA while minimizing CPU involvement and handling interrupts.

Input: Device address, memory address, transfer size
Output: Data transferred successfully

Pseudocode

BEGIN

1. Initialize DMA controller

2. Set source = I/O device

3. Set destination = memory

4. Set transfer_size

5. Configure DMA for I/O-to-memory transfer

6. Enable DMA interrupt

7. Start DMA transfer

8. WHILE DMA transfer is not complete

CPU performs other tasks

END WHILE

9. WHEN DMA interrupt occurs

Check DMA status

IF transfer is successful THEN

Mark transfer as complete

ELSE

Report transfer error

END IF

10. Stop DMA controller

END

## OUTPUT

### DMA Data Transfer

DMA Controller Initialized

Source      : I/O Device

Destination : Main Memory

Transfer    : Started

CPU         : Performing Other Tasks

DMA         : Transfer Completed

Interrupt   : Received

Status      : SUCCESS

### Result

Data is transferred directly from the I/O device to memory with minimal CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

Aim: To prioritize interrupts from multiple devices and execute the appropriate service routine.

Input: Interrupt requests from multiple devices
Output: Appropriate ISR execution

### Pseudocode

BEGIN

1. Initialize interrupt controller

Assign a vector to each device

Assign priority to each device

Enable interrupts

WHILE system is running

IF interrupt request occurs THEN

Identify interrupt source

Read interrupt vector

Determine priority

Select highest-priority interrupt

Save CPU context

Jump to corresponding ISR

Execute ISR

Clear interrupt flag

Restore CPU context

END IF

END WHILE

END

### Result

The highest-priority interrupt is serviced first using its corresponding vectored service routine.

OUTPUT:

### Vectored Interrupt Handling

Interrupts Received : Device 1, Device 2, Device 3

Highest Priority    : Device 2

Vector              : 002

ISR Executed        : Device 2 ISR

Interrupt Cleared

Status              : SUCCESS.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

Aim: To dynamically select the appropriate I/O technique based on latency and bandwidth requirements.

Input: Latency requirement, bandwidth requirement
Output: Selected I/O technique

### Pseudocode

BEGIN

1. Read latency_requirement

2. Read bandwidth_requirement

3. IF latency_requirement = HIGH THEN

Select MEMORY_MAPPED_IO

4. ELSE IF bandwidth_requirement = HIGH THEN

Select MEMORY_MAPPED_IO

5. ELSE

Select IO_MAPPED_IO

END IF

6. Configure selected I/O method

7. Perform data transfer

8. Monitor performance

END

### Result

Memory-mapped I/O is selected for demanding low-latency or high-bandwidth applications, while I/O-mapped I/O is used for simpler I/O requirements.

OUTPUT:

### I/O Method Selection

Latency Requirement  : HIGH

Bandwidth Requirement: HIGH

Selected I/O Method  : MEMORY-MAPPED I/O

Reason               : Low Latency and High Bandwidth

Status               : SELECTED

4.Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

Aim: To manage multiple devices requesting access to PCIe, USB, and Thunderbolt interfaces while providing fair bus access.

Input: Device requests
Output: Granted bus access

### Pseudocode

BEGIN

1. Initialize PCIe, USB and Thunderbolt controllers

2. Initialize device request queue

3. Assign priority to each device

4. Set current_device = 1

5. WHILE system is running

Check all device requests

IF high-priority request exists THEN

Grant bus to high-priority device

ELSE IF normal requests exist THEN

Select next device using round-robin

Grant bus access

END IF

Transfer data

Release bus

Move to next requesting device

END WHILE

END

### Result

The arbitration algorithm provides controlled and fair access to the communication bus while giving priority to urgent requests.

OUTPUT:

### Bus Arbitration

Devices Requesting Bus:

PCIe → USB → Thunderbolt

Highest Priority : PCIe

Bus Granted      : PCIe

Transfer          : Completed

Next Device       : USB

Bus Status        : FAIR ACCESS

5.Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Aim: To use polling for low-speed devices and DMA with interrupts for high-speed devices.

Input: Device type and data rate
Output: Appropriate I/O method

### Pseudocode

BEGIN

1. Detect connected device

2. Measure device data rate

3. IF data_rate < LOW_SPEED_LIMIT THEN

Select POLLING

WHILE device is active

Check device status

IF data is available THEN

Read data

Store data in memory

END IF

END WHILE

4. ELSE

Select DMA

Configure DMA controller

Set source and destination

Start DMA transfer

Enable DMA interrupt

WHILE transfer is active

CPU performs other tasks

END WHILE

WHEN DMA interrupt occurs

Check transfer status

Process completed data

END WHEN

END IF

END

### Result

Low-speed devices are handled using polling, while high-speed devices use DMA and interrupts, reducing CPU overhead and improving overall I/O performance.

OUTPUT:

### Hybrid I/O

Device 1 Data Rate : Low

I/O Method         : POLLING

Device 2 Data Rate : High

I/O Method         : DMA + INTERRUPT

DMA Transfer       : Completed

Interrupt          : Received

CPU Usage          : MINIMIZED

Status             : SUCCESS

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |