Q1.Signed 8-bit Addition and Subtraction:

Given:

+45 and −23

Binary conversion:

+45 = 00101101

+23 = 00010111

−23 using 2’s complement:

1’s complement = 11101000

+1 = 11101001  Addition:

00101101   (+45)

+ 11101001   (-23)

-----------

00010110

00010110 = +22

Subtraction:

(+45) − (−23) = 45 + 23

00101101

+ 00010111

-----------

01000100

01000100 = +68

Overflow:

No overflow occurs because both results are within the 8-bit signed range−128 to +127.

Answer:

Addition = +22, Subtraction = +68.

Q2.4-bit Carry Look-Ahead Adder:

A Carry Look-Ahead Adder (CLA) calculates carry signals in advance, reducing addition delay.

For each bit:

#   Generate:  Gi = Ai · Bi

  Propagate:  Pi = Ai  Bi    Carry equation:  Ci+1 = Gi + PiCi

For a 4-bit CLA:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1      + P3P2P1G0 + P3P2P1P0C0 The sum is:

Si = Pi  Ci

Comparison

| Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- |
| Carry moves bit-by-bit | Carries calculated in parallel |
| More delay | Less delay |
| Simple circuit | More complex circuit |
| Slower for large adders | Faster for large adders |

Conclusion

CLA reduces propagation delay because carries are calculated simultaneously. Hence, it is useful in high-speed processors and ALUs.

Q3. Booth’s Algorithm: −6 × +5:

Use 4-bit 2’s complement.  	Multiplicand M = −6 = 1010

- Multiplier Q = +5 = 0101

- Accumulator A = 0000

- Booth bit Q−1 = 0

Rules:

| Q0 Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 11 | No operation |
| 01 | A=A+M |
| 10 | A=A-M |

Steps

| Step | Q0Q−1 | Operation | A |
| --- | --- | --- | --- |
| 1 | 10 | A = A − M | 0110 |
| 2 | 10 | A = A − M | 1100 |
| 3 | 01 | A = A + M | 0110 |
| 4 | 10 | A = A − M | 1100 |

After every operation, an arithmetic right shift is performed on A, Q, Q−1.

Final 8-bit product:

AQ = 11100010

11100010 in 2’s complement = −30.

Therefore:

−6 × +5 = −30

## Advantages of Booth’s Algorithm

- Handles positive and negative numbers directly.

- Uses 2’s complement arithmetic.

- Reduces the number of additions/subtractions when consecutive 1s occur.

- Suitable for signed multiplication in processors.

Q4. Restoring and Non-Restoring Division: 13 ÷ 3:

Given:

- Dividend = 13 = 1101

- Divisor = 3 = 0011

Expected result:

13 ÷ 3 = 4 remainder 1 A. Restoring Division

Basic procedure:

- Shift the partial remainder and quotient left.

- Subtract the divisor.

- If result is negative, restore the previous remainder and put quotient bit = 0.

- If result is positive, keep it and put quotient bit = 1.

- Repeat for all bits.

For 1101 ÷ 0011, after four iterations:

Quotient  = 0100 = 4

Remainder = 0001 = 1

The important feature is that a negative partial remainder is restored by adding the divisor back.

## B. Non-Restoring Division

In non-restoring division:

- If the partial remainder is positive, subtract the divisor.

- If it is negative, add the divisor in the next step.

- Restoration is therefore avoided after every unsuccessful subtraction.   	At the end, if the remainder is negative, the divisor is added once.

For 13 ÷ 3:

Quotient  = 0100 = 4

Remainder = 0001 = 1

Comparison

| Restoring | Non-Restoring |
| --- | --- |
| Restores negative remainder | Does not restore immediately |
| More operations | Fewer operations |
| Simpler concept | More efficient |
| More delay | Less delay |

Conclusion:

Non-restoring division is generally faster because it avoids repeated restoration operations.

Q5. IEEE 754 Representation of −13.25:

Given decimal number:

−13.25

## Step 1: Convert to binary

Integer part:

- = 1101

Fraction:

0.25 = 0.01 Therefore:

13.25 = 1101.01 Normalize:

1101.01 = 1.10101 × 2³

## Single Precision — 32 bits

IEEE 754 single precision contains:

- Sign: 1 bit

- Exponent: 8 bits

- Mantissa: 23 bits

For −13.25:

Sign = 1

Exponent = 3 + 127 = 130

130 = 10000010

Mantissa = 10101000000000000000000 Therefore:

1 | 10000010 | 10101000000000000000000 Double Precision — 64 bits

Double precision contains:

- Sign: 1 bit

- Exponent: 11 bits

- Mantissa: 52 bits

Bias = 1023

Exponent = 3 + 1023 = 1026

1026 = 10000000010

Therefore:

1 | 10000000010 | 1010100000000000000000000000000000000000000000000000 Floating-Point Addition

When two floating-point numbers are added:

- Compare their exponents.

- Align the binary points by adjusting the smaller exponent.

- Add/subtract the significands.

- Normalize the result.

- Round the result according to the IEEE 754 rounding mode.

- Store the final sign, exponent and mantissa.

## Important issues

- Normalization keeps the significand in the required form.

- Rounding may be necessary when the result has too many bits.

- Precision loss can occur when numbers have very different exponents.   	Double precision provides greater precision than single precision.

Final: IEEE 754 provides a standard way to represent and perform arithmetic on real numbers using sign, exponent, and mantissa fields.

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |